Name  __________________________

ICT Logic Gate Pretest

Draw the logic diagram of the following boolean expressions (be neat and use a template):

1.  (A + B)(C + D) = Y														2.  N⋅X+(A⋅B)=Y

3. (C⋅W⋅P)+(V+W)=Y												4.  (A⊕B)+(C⋅D)=L

Write the boolean expression for each of the following logic diagrams:

Y = __________________________							Y = __________________________

Y = __________________________							Y = __________________________

9)		A NOR gate can be constructed by inverting a(n)	__________________ gate.

10)	A XNOR gate can be constructed by inverting a(n) __________________ gate.

11) 	LED is an acronym for	__________________  __________________  __________________.

12)	DIP is an acronym for _______________  _______________  _______________.

13)	RAM is an acronym for  _______________  _______________  _______________.

14)	ROM is an acronym for  _______________  _______________  _______________.

15)	IC is an acronym for  _______________  _______________.

16)	A  _______________  gate has a unique output of low (or 0) when both inputs are high (or 1).

17)	A  _______________  gate has a unique output of low (or 0) when both inputs are low (or 0).

18)	A NOR gate has a unique output of  _______________  when both inputs are _______________.

19)	The  _______________  and  _______________  gates have no unique outputs.

For questions 20-22 choose from the following gates (AND,  OR,  XOR,  NAND,  NOT)

20)	The "either but not both gate" is the _______________ gate.

21)	The "all or nothing gate" is the _______________ gate.

22)	The "any or all gate" is the _______________ gate.

23)	Complete the following truth table:

| A | B | A⊕B | AB | AB | A + B |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |
| 0 | 1 |  |  |  |  |
| 1 | 0 |  |  |  |  |
| 1 | 1 |  |  |  |  |

24)	Complete the truth table for the following:							25)  Label the legs of this IC.

26)	The output of gate  1  is _____.

27)	The output of gate  2  is _____.

28)	The output of gate  3  is _____.

29)	A  NOR  gate performs the logic function  ________.

a)  A + B		b) A+B		c)  AB			d) AB			e)  A⊕B

30)	A  NAND  gate performs the logic function ________.

a)  A + B		b) A+B		c)  AB			d)  A	B		e)  A⊕B

Name each of the following gates (choose from:  NOT, OR, XNOR, XOR, NAND, AND, NOR)

_____     		_____   		 	_____    		 	_____    			_____    			_____ 		 	_____

Simplify the following bit-wise expressions:

31)	10101⋅11011⊕10011=

32)	11000+10101⋅01010=

33)	00110⊕10011⋅11101+00100=